Geography 303: Field Study in Environmental Geography

Lab #8: Stream Profile, Streamflow, and Tucson’s Floodplain

In this group lab we are going to be collecting data and observations from a river bed. You will use the data in some calculations to determine the volume of water that could flow past a given point and will conduct qualitative observations to answer some basic questions about the river its floodplain.

Floodplains are areas near rivers or streams that experience recurring inundation by water.  The return period is the expected frequency of an event expressed as a percent likelihood that it will occur in any given year. For example, a 100-year floodplain is the area that has a 1 percent chance of flooding in a given year. To calculate the percent likelihood, divide 1 by the return period:

Thus, 1/100 = .01 = 1%

Streamflow is the volume of water that flows through a channel per unit time. To determine streamflow, we can use Manning’s formula:

Q = (k/n) * (A/P)2/3 * A * S1/2

Where:

Q = volume of flow in ft3/s

k = 1.49 ft1/3/s

n = a coefficient that depends on the material in the channel, which we approximate as 0.04 for this brushy river bed

A = cross-sectional area of flow in ft2

P = wetted perimeter in ft, meaning the length of the transect along the river bed that is covered by water

S = slope of the river. In this case you will assume that S=0.01

You will be measuring A and P, then calculating Q based on a given flow depth. Use the excel worksheet “Lab 8_StreamProfile Data Template.xlsx” to help you.

Lab Deliverables:

Each group will hand in one lab assignment. This will include written answers to the questions below, an annotated aerial photo of the study site, and the completed excel template with all data, calculated streamflow, and the stream profile graph. Include the stream profile graph in your write-up and the values you calculate for streamflow. Also copy and paste (and annotate) the aerial photo of the study site into your write-up. For the floodplain section, we encourage you to use screen captures to document the online GIS map that you’ll be analyzing.

Part I, Data Collection:

A stadia rod is used to measure the relative change in elevation between two or more points.  I will demonstrate the proper technique to determine elevation changes. You will use a string and level or clinometer to make sure your measurements are consistent.

You have been given a satellite photograph of Santa Cruz River. Your group will find a place where you can safely take measurements from one side to the other.

Procedure:

- On the satellite image below, draw a line that corresponds to the transect you have chosen.

- Determine the eye-level height of one member of the group. This person should determine all of the heights on the Stadia rod throughout the lab.

- Write this information here:

  - name______________________; eye height (ft)________________

- Begin your transect on the west side of the river bed. The first position should be against the steep wall, and this position is marked as zero (0) in terms of distance and elevation. Each subsequent point will be relative to your starting elevation (of zero). Make sure that your transect stays along a straight path that is perpendicular to the river bed, heading generally west. To accomplish this you can use a compass and make sure you hold an azimuth all the way across, or pick a tall object in the distance (like a cell tower or building corner) that the person taking measurements can ensure is always directly behind the stadia rod at each measurement.

- Every 3 ft of horizontal distance, record the height of the stadia rod (from the last position).  The height of the stadia rod is recorded as the number at the eye level of the measurer. Be sure to use a level or string to ensure a level reading every time.  Record each measurement in the data template below (use a notebook if you run out of space).

- When you reach the far bank, make sure to record a last point at least 3 feet up from the river bottom. This will ensure that you have enough rise on the opposite end to complete your stream profile. Don’t climb on the steep bank – it’s dangerous!

- As always in the field, be observant. Look at the environment you’re in and make notes of interesting or seemingly important features related to streamflow, flooding, and environmental conditions. Take photos of your group working and the surroundings to help you remember these features later and better illustrate your explanations to the questions below.

Part II, Stream Profile and Stream Flow:

- Open the excel template, “Lab 8_StreamProfile Data Template.xlsx”. You will see in the spreadsheet that there are yellow cells and blue cells. The yellow cells require you to input specific data, as described below. The blue cells contain formulas that will calculate the important variables for you. The sheet labeled “Stream Profile” will create a graph of your measured stream profile for you.

- Enter the eye height of your stadia rod measurer in the yellow cell B6.

- You have collected slope distance and stadia rod measurements for every three (3) feet of horizontal distance across the Santa Cruz River. Enter these data into the spreadsheet in their appropriate columns.

  - If you have a longer transect than 201ft horizontal, then add more rows, and drag down the formulas within the blue cells to the right.

  - If you have fewer than 201 ft horizontal, delete the cells of all columns below your last filled-in row.

- Once you’ve entered your recorded measurements, you should see the columns for Elevation Change and Relative Elevation fill in with values. Elevation change is the difference between each measurement you made along the transect, with the first point (0) being the fixed elevation that everything is compared against to start out. The Relative Elevation is the elevation of the river bed relative to its lowest point, as shown in the cell for “Minimum Elevation” that determines the lowest point along your transect. Once the lowest point is determined, all other points are set as relative to that, and the profile for the stream becomes positive. This allows you to determine the points at which 2 ft of water would rise to.

- For the worksheet to calculate A (the cross-sectional area), you have to identify the distances along your transect where the 2 ft water line would be. This will be the width of the stream with water at a depth of 2ft (see the image below). You can see the width visually in the graph on the Stream Profile worksheet. First find the point (horizontal distance) on the left side of your graph where the relative elevation is 2 ft. Type this distance measurement into the yellow cell B7. Next, find the second point on the right side of your graph where the relative elevation reaches 2 ft. Type this value into the yellow cell B8. Once you’ve done this, you should see a value for the A parameter in cell I18.

- Next, you need to calculate the wetted perimeter (P) for the equation, given 2 ft of water. To do this, go back to your data in the columns and find the rows that most closely correspond to the horizontal distances you determined for the cross-sectional area in step 5. What you need to do now is calculate the total slope distance of the river bottom given 2 ft of water. So you will sum all of the slope distance measurements between the 1st and 2nd points of where the water level is 2 ft.

  - To do this, use the SUM() function in cell B9. If for instance, your 2 ft water level occurs from distances of 12 ft to 21 ft, you would type =SUM(B17:B20).

- Once you have calculated both the cross-sectional area of the stream and the wetted perimeter, you have completed the necessary steps for calculating the actual streamflow. Your streamflow value should now be present in the blue cell I23.

Questions

- Describe your stream profile. Does the graph match what you thought it should look like in the field? Why does it have the shape that it does?

- Given the amount of vegetation on the higher banks of the stream profile, how high do you think water commonly gets when the river is flooded along your transect?

- How high would the water need to get to fully inundate your transect and fill the Santa Cruz from wall to wall?

- What was the final streamflow rate? What effect does changing the roughness coefficient (n) have on streamflow and why? What effect does changing the river slope (S) have on streamflow and why?

- If you were to change the water depth from 2 ft to 3 ft, would the resulting streamflow be accurate? Why or why not?

- If you were to take more or fewer measurements along your transect, how would your profile change?

Part IV, Tucson Floodplain:

For this part answer the following questions using the observations and your own research.

- Based on the debris pattern and erosion patterns, which way do you think the river flows?  (north, east, south or west). Why do you think this?  (you may use your photos as part of the answer to this question)

- Go to this website:  http://maps.tucsonaz.gov/pdsd/

On the left side under table of contents, go to ‘engineering’ and then to FEMA Floodplain Current. Using the + button, zoom into the portion of the river we just visited.  Use the ‘hand’ button to navigate around town if needed.

  - TIP: Hover over the table of contents again, and select the legend tab, and press the thumbtack button in the upper left of the window to keep the legend on the screen for easy viewing.

- What are the odds that the neighborhood along Riverside Rd will be flooded in a given year?

- Why is the flood area split between the east and west sides of I-10?

- The green lines in the middle of the river show the base elevation during flood. From these values, which way do you think the river would flow during a flood?

- Are the university, downtown, and some old Tucson neighborhoods (like El Presidio) in floodplains? Do you think these areas were built with knowledge of the floodplains? Why or why not?

- Find one other area of interest to your group (a house, favorite restaurant, anything) and see if it lies in a floodplain. Where is it? If it is in a floodplain, what is the percent chance of a flood in any given year? What is the return period?

Aerial Image of the Study Area. Draw in where you conducted your transect.

Speedway

| Horiz. Dist. | Slope Dist. | Measurement |
| --- | --- | --- |
| 0 | 0 | 0 |
| 3 |  |  |
| 6 |  |  |
| 9 |  |  |
| 12 |  |  |
| 15 |  |  |
| 18 |  |  |
| 21 |  |  |
| 24 |  |  |
| 27 |  |  |
| 30 |  |  |
| 33 |  |  |
| 36 |  |  |
| 39 |  |  |
| 42 |  |  |
| 45 |  |  |
| 48 |  |  |
| 51 |  |  |
| 54 |  |  |
| 57 |  |  |
| 60 |  |  |
| 63 |  |  |
| 66 |  |  |
| 69 |  |  |
| 72 |  |  |
| 75 |  |  |
| 78 |  |  |
| 81 |  |  |
| 84 |  |  |
| 87 |  |  |
| 90 |  |  |
| 93 |  |  |
| 96 |  |  |
| 99 |  |  |
| 102 |  |  |
| 105 |  |  |
| 108 |  |  |
| 111 |  |  |
| 114 |  |  |
| 117 |  |  |
| Horiz. Dist. | Slope Dist. | Measurement |
| 120 |  |  |
| 123 |  |  |
| 126 |  |  |
| 129 |  |  |
| 132 |  |  |
| 135 |  |  |
| 138 |  |  |
| 141 |  |  |
| 144 |  |  |
| 147 |  |  |
| 150 |  |  |
| 153 |  |  |
| 156 |  |  |
| 159 |  |  |
| 162 |  |  |
| 165 |  |  |
| 168 |  |  |
| 171 |  |  |
| 174 |  |  |
| 177 |  |  |
| 180 |  |  |
| 183 |  |  |
| 186 |  |  |
| 189 |  |  |
| 192 |  |  |
| 195 |  |  |
| 198 |  |  |
| 201 |  |  |