Geography 303: Field Study in Environmental Geography

Assignment 3:  Compass use, estimating distance, angles and mapping

Pacing  (work individually)

- Using your normal stride, walk along the 50 m tape and count the number of steps that you take.  Write this number here: _____________________

- Walk back along the 50 m tape in the other direction to where you started and count the number of steps you take this time.  Write this number here: _______________

- Average these two values and write the number here: ____________

- Divide 50 by the average number of strides (step 3) to determine the number of meters per stride.  (If you took 75 strides, you would come up with 50/75 = 0.667 m/stride.)  Write this number here: _______________

- Multiply the number of paces that you take over a given distance by this number to come up with a simple estimate of distance.  So, to test yourself, walk back along the 50 m tape counting your strides.  Take this number of strides and multiply by the number you calculated above in #4.  This should give you something in the range of 50 m (otherwise, you should repeat it!)

- Finally, estimate the distance between the two objects that you are assigned using pacing.  After you make your calculation, write the numbers below.

Tapes  (work in pairs)

- The basic idea is simple – values are measured from the metal hinge, so place this part of the tape (not where the black tape begins or ends) at the start point.  There are several things to keep in mind:

  - Tightness:  if you’re running the tape along the ground, pull the tape taut so that there isn’t any slack (overestimating the distance)

  - Sag: if you’re holding the tape off the ground, make sure there is as little sag as you can (to prevent overestimating distance)

  - Make sure of the units you’re using (metric or English) – some tapes have both measures

- When you’re ready, measure the ground distance between the two objects that you are assigned.  Measure this distance and write the values below.  The tape measure is in meters and centimeters, but you should record your distance to the nearest hundredth of a meter.  So, if your measurement is 7m 37 cm, your measure would be 7.37m.

Taking a bearing (work in pairs)

- If necessary, re-familiarize yourself with the compass.

- Next, take three bearings relative to the azimuthal directions below.  For each direction identify the best orienteering point of reference.  Stand at the flag next to the “Communication Building” Sign.  Be detailed and precise in terms of what you describe at each bearing (“palm tree” is too vague).

  - 28 degrees

  - 175 degrees

  - 230 degrees

- Identify the azimuthal direction for the landmarks given below (stand at the small flag).

  - “Arizona State Museum” Sign

  - Top of Old Main Tower

  - US Flag Pole

Measure a slope using your compass’ slope needle

- Measure the slope of the grade that is identified (by a flag) between two levels/terraces. Measure this slope 5 times in slightly different places and take the average value.

Write all 5 values down here: 1) _________

2) _________

3) _________

4) _________

5) _________

Write the average slope value here: _____________

## (Calculate a tree height- see instructions below)

## Angle of Elevation Method

## Tree Height = [TAN (Angle of Clinometer) x (Horizontal Distance to Tree)] + Eye Height

## Fixed Angle of Elevation Method

If the angle to the top of the tree 45º, then its tangent is 1. So walk out until the angle is 45º, then add the distance to the tree to your eye height to get the tree’s height.

- What is the angle between where you stand and the top of the smaller tree 20 meters away from you? (compute the tree height).

- Move away from the larger tree until your angle is 45º. Measure the distance to the tree and add in your eye height.  What is height of this tree?

Orienteering (work in pairs)

- To practice your compass handling skills, we’ll be playing a quick game of “Closest to the pin.”

- Proceed to the orienteering start point marked by the pink flag.

- From the flag, figure out the location of the first orienteering point based on the compass direction and distance given below.  Place a flag in the ground at this spot.

- Next, proceed to the second location from the flag that you just placed based on the compass direction and distance given below.  Place a flag in the ground at this spot.

- Finally, proceed to the third location from the second flag that you just placed based on the compass direction and distance given below.  Place a flag that has your group # in the ground at this spot.

- Now pick up the intermediate flags, give them to your instructor and let him know that you are finished with this problem.

- Draw your path from the Orienteering question onto this aerial photo.  Be sure to use the 10m scale bar provided (on brick walkway).  North is the top of the photograph.

- Your assigned distances and compass headings are:

  - From start flag to the first point:		12.3 m at 190 degrees

  - From first point to second point:		8.2 m at 310 degrees

  - From second to third point:			4.0 m at 245 degrees

  - From third point to final flag:			16.7m at 170 degrees

Turn in your equipment before leaving.

N (0°)

10 m